# Training package for FS2004

The intention of this package is to help you understand how we use the flight simulator for soaring, to help you to become familiar with the instruments and tools, and to have a bit of fun. I have tried to answer the questions that are frequently asked by people as they are learning to soar using the simulator. If you have more questions, or something is not clear please ask on the SOAR message board and someone will try and help.

This package is written for people who do not have any experience in flying gliders. If you are a glider pilot you can probably skim some of this information. This package does assume you have some basic idea on how to run the simulator.

This package includes a scenery file which contains some lift, a setup flight to help you get started, data files for the GPS, plus a few extras, to be covered later. A modified copy of the winch launch script by Peter Lürkens is also included. The copy here has the cable break feature disabled to ensure you get flying

This package does not include any aircraft, instruments or other tools, you can obtain them from the links below, or via the SOAR site, they are all free.

# Things to download

## Aircraft and gauges

You will need a glider fitted with at least the CAI instrument set (more on this later). I would recommend the ASW28 by Peter Frank as a good aircraft. It is available at …, this package has been written using the ASW28 but you should be able to use any of the aircraft tat are available as long as they have the CAI instruments fitted.

Other gliders currently available (at least the ones I know about) for fs2004 are:

Grob 102 Astair-CS, available at http://www.fszwever.com/

Various aircraft by Wolfgang Piper available at http://home.t-online.de/home/w.piper/homepage.htm, Note that some of the Wolfgang’s are fitted with old versions of the CAI instrument set and the panels will need to be updated to use them successfully with FS2004. For older style aircraft that do not have modern instruments on them it is possible to use the CAI pop up panel by Kris Feldmann (http://www.rotted.com/)

You can get flying with just this package and an aircraft, but I would recommend you download at least the Viewer from the list below before you go too far.

## Viewer

The viewer allows you to create tasks (a route you intend to fly) which can be downloaded to the GPS. It also allows you to view and save your flight data after your flight, which you will need to do if you want to fly in any races of any of the certificate flights.

The viewer is available at http://www.fszwever.com/. There are also tutorials available at the same place.

## Installation

Firstly unzip the contents to an empty folder somewhere.

You will need to copy the following directories into the locations below, if you already have some of these directories just copy the files into the correct locations. “…\fs9” is the directory where you installed the flight simulator, this will be somewhere like “C:\Program Files\Microsoft Games\Flight Simulator 9”

Copy the “SOAR flights” directory to “…\Flight Simulator 9\Flights” directory

Copy the “SOAR Scenery” directory to the “…\Flight Simulator 9\Addon Scenery” directory.

After you start the simulator you will need to add and activate the scenery folder …\Flight Simulator 9\Addon Scenery\SOAR Scenery\Thermal BGLS from the settings screen by pressing the “Scenery Library” button then “Add Area” and select the folder. Under Settings->Display->Scenery you must have Autogen Density set to at least sparse.

If you want to get flying straight away, without using the viewer to load the GPS, copy “…\Flight Simulator 9\Addon Scenery\SOAR Scenery\GPS_DAT\short1.dat” to “…\Flight Simulator 9\gps-nav.dat”, this will load the GPS with the short1.dat data file.

# Loading the GPS with the viewer

Start FSZweverView 2. Note that FSZweverView 2 must be installed in the directory fs9 root directory to work correctly.

From the menu select “Load->any *.DAT file”. Navigate to “…\Flight Simulator 9\Addon Scenery\SOAR Scenery\GPS_DAT” and select short1.dat.

This file contains a short course on the south island of New Zealand. The course should be shown in the viewer. The red triangle is the task (course) to be flown. The circles with numbers next to them are waypoints. The task details are shown in the top left corner.

From the menu select “Send->to GPS-NAV”, then press OK. That is it.

# First flight

This is a quick guide to get you up and flying, see sections below for more details.

Start the simulator and select the training_2k4 flight from the “SOAR flights” preset flight. If all is installed correctly you should be on the ground with clouds in the sky. Note that the cockpit has been pulled down in the training_2k4 flight. To see the other instruments (you wont be needing them) drag it up.

Turn on the GPS (the square instrument with a grey display) by pressing the “go” button. Information should appear on the display. To select the task press the right arrow 6 times, the display should display “Task Selection”. Press the Go button twice, the display should display “Task A, Declare? No”. Press the down arrow to change it to Yes and then press go. The display should display “Task A, Start? No”. Again press the down arrow to change it to Yes and then press go. The display should show “on task” along with some other information. You would normally do this procedure in the air, before starting the task but for the first flight it is simpler to do it on the ground.

Turn on the glide computer (the round instrument with a grey display) by turning the volume control, turn the volume up a little so you can here it (it will stop making a noise after a few seconds). We will ignore the glide computer display for this first flight.

Next thing is to perform your pre flight checks, at least look at the wings and ensure you spoilers are closed. Now we are ready to launch. First press the f1 key followed by the f4 key to trigger the winch script to start (this closes and opens the throttle). You should see and hear a “Cable on please” request. Press “Ctrl [“. There should then be “Open – Close, All Clear Above and Behind, Take-up slack!, Cable live, All out”, the glider will then start to move. When a safe flying speed is reached, around 60kts, ease the glider into the air and start climbing. Maintain a safe airspeed during the climb using pitch, between 50kts and 60kts. There is a cloud in the example flight, if you keep that so that you can just see it all over the top of the canopy your pitch will be about right. At the end of the climb the final message will disappear and the winch will disconnect. Push the stick forward and start flying. Generally pitch is used in a glider to control speed, try and maintain a speed of between 70 and 90kts when not climbing for this flight.

You can retract the wheel now by pressing “g”.

To gain height you will need to find some lift. All of the clouds in the example flight have lift under them, there is also some other lift about. To gain height find some lift and fly through it. To stay in the lift you can fly in circles, this takes a little practice but is part of the art of gliding. You should reduce your speed when climbing, try to fly at about 50kts when climbing for this flight. The glide computer will beep when you are climbing, the variometer will show your rate of climb or decent. Note that you will drift downwind while circling and will need to fly back upwind at times to stay in the lift.

So lets start flying the task. The GPS had a arrow in the middle of it that points either left or right, that is the direction you need to turn to head towards the current turn-point, the longer the arrow the more you will need to turn. So start to head towards the first turn-point. When you get within 2km of the turn-point the display will show that you are “Close To” the waypoint, when you are within 0.5km it will show “Arrival”. Note you distance from the current turn-point is shown at the bottom of the GPS. Once you have arrived at a turn-point you can select the next turn-point by pressing the right arrow twice and then the go button.

Hopfully that should be enough information for you to fly around the first small training course. Once you reach the finish you can land back at the airfield which is to the north east of the last waypoint.

# Viewing and saving flight data with FSZWeverView

After land you may wish to examine and save the data that was recorded by the GPS during the flight. If you fly any certificate flights, races, etc you will need to save the data as proof that you flew the task correctly. A word of warning, each time you switch on the GPS it will destroy any data from a previous flight so do not turn the GPS off while flying or start another flight without saving any data that you need.

Viewing and saving data is simple. After your flight open “FSZWeverView” select “Load->From GPS-NAV” from the menu. The main window should show all the turn-points that were loaded into the GPS as circles, the task that you had programmed, and the task that you flew. The Profile window will show your altitude throughout the flight.

To save the flight press the first disk icon on the toolbar and select a file name to save it in.

For more information on using FSZWeverView see the tutorials on Max’s site.

Once you have saved your data you can load another flight and fly some more. The package has another short flight a couple of slightly longer triangular flights and a silver distance flight (see below) in it that you can fly. There are many other flights, in various parts of the world, available at the SOAR site, see links section below.

# Silver Certificate

The silver.dat file is a short flight starting with the training flight and heading west to another airport, it is slightly harder than the other flights. You will need to gain 1000m in height during this flight (difference in height between a high point and a previous low point) to claim the silver certificate. Once you have flown the flight you should save the flight data and .dat into a zip file. The files should be named "Silver-(your SOAR#)(.log, .dat and .zip) and sent to director@virtualsoaring.org.

# Flying a glider

## General

This is a fairly basic guide on how to fly a glider on cross country courses, it is not intended to be a comprehensive course on fly gliders, there is plenty of material available to help you to learn more.

## Polars

First a little bit of  basic theory. A glider when flying is always descending relative to the air it is flying in. The rate at which it descends depends mainly on the speed at which it is being flown. The relationship between the decent rate and speed for a particular glider is generally called the polar for the glider. This will be different to each type of glider and for the amount of weight the glider is carrying (we will ignore other factors such as how clean the glider is). A typical polar (for the ASW28) is shown below.

## Cruising

As can be seen the faster the glider fly’s the higher the decent rate. When flying locally around an airfield it may be desirable to fly slowly so as to minimize the amount of time need to be spent climbing. When flying cross country the important thing is often the average speed, as it is possible to fly further in a certain amount of time, or win races. To achieve the maximum average speed it is necessary to fly at a speed between lift that gives the maximum average ground speed from the top of one area of lift to the top of the next. This is where the glide computer comes into play, plus some experience. As a very rough guide, on a course where there is reasonable lift flying at around 90kts in the ASW28 is reasonable. 70kts is about the speed that will get you the greatest distance with the least loss of height in the ASW28. This is know as best L/D speed and is a useful speed to know if you need to travel a long distance without lift.

## Climbing

The first thing to be aware of is where to find lift. The simulator provides two types of lift, thermal lift, which simulates lift caused by the sun heating the ground, which produces rising warm air. This type of lift is dependent on the time of day, being strongest around noon. Thermal lift often causes clouds to be formed as the rising air reaches the dew point, This is often simulated by placing a cloud over a thermal, as in the training flight.

The second type of lift is wind dependent. This is mostly used to simulate the wind blowing onto the side of a hill, which causes the air to rise to go over it, and is generally called ridge lift. Wind dependent lift id dependent on the strength of the wind and the relative angle of the wind to the lift, i.e. there is more lift when the wind is blowing directly onto the face of a hill than when it is blowing at an angle to it. As a rough guide a moderate wind in the simulator generally provides good lift. Most of the scenery has been designed for a westerly breeze, but check the package details.

Now you have some idea where a course designer may have placed the lift you will want to climb in it as quickly as possible (too keep that average speed up). Most of the simulated courses that we have at present do not model the way real lift changes in strength depending where you are in it, so simulated climbing is a little simpler than in real life. For thermal lift you basically you need to stay within the lift while flying at a speed that will make you climb the fastest. If you look at the polar again you will see that there is a flat (ish) section at the top of the polar, just above stall speed, you want to be flying in this speed range to minimize the gliders decent rate. To stay within the lift you generally fly in circles. The basic technique that I use is, once I reach a thermal I continue flying for a few seconds, until I think Im a little way past the centre of the thermal (say 5-10 seconds) and then start turning. You need to judge the angle at which you bank the glider to make the maximum rate of climb, too shallow and you will fly out of the thermal, too steel and you will not climb so quick. Around 30deg is a reasonable starting point. While thermaling you are bound to fly out of the thermal from time to time. The thing to do then is to decide whether you wish to waste the time and find the thermal again, and if you do to fly back into it. Bare in mind that the simulated thermals generally do not slant with the wind, so you are likely to drift out of them with the wind, this can be corrected by occasionally straightening up when pointing into the wind for a few seconds (more practice). One thong that I find useful is to use the virtual cockpit and the hat switch to keep an eye on the cloud. Switch back to the 2D cockpit immediately gets me looking forward straight away (I have the second button on my joystick to rotate views backwards so that I can jump between the two views quickly).

Flying ridge lift is a little different at the ridge lift often runs for many kilometers and often does not extend a great height above the ridge top. Generally you want to keep in the lift and fly at a speed that will keep you at a constant height above the ridge top.

## Landing

When it comes to landing gliders use spoilers to control the rate of decent. The spoilers are generally just plates that come out of the wings, look out of the window when you open them. If you have a throttle lever then you can assign the spoilers to it. You should reverse the axis as the spoilers should be closed when the control is forward. To land you should set up a reasonably steep decent and then use the spoilers to control your decent rate and pitch to maintain speed. You should aim to have the spoilers about 50% deployed for the final approach. The “/” key will operate the spoilers if you do not have a throttle leaver (no subtle movements here). Don’t forget to lower the wheel too.

# The winch launch

This package includes a slightly modified version of the winch launch script by Peter Lürkens. this is what you used to take off in the example flight. If you want to use the winch launch for other flights you should download the original script from http://www.virtualsoaring.org/utilities.html (The script included in the package has the cable brake feature disabled to ensure you get airborne.)

To use the winch launch with any flight you have a couple of choices. The option that is suggested is to take the example flight that came with the winch launch and the move to another location and save the flight.

Another method is to open the .FLT file of a flight that you have saved in an editor (e.g. notpad) and add the line “ablscript="Winch14"” to the [main] section. See the example flight that came with the winch launch as an example. Note that when you save a flight fs9 saves it in “My Documents\Flight Simulator Files”.

Add other bits about default aircraft etc

# Using the glide L-NAV computer

The main instruments used in flight are the Cambridge Instrument set, comprising of the GPS, the glide computer and the electronic variometer. These instruments are modeled on a real set of instruments. I covered the essential use of the GPS and variometer in the first flight section. I am only intending to give a basic working guide for the glide computer here. For more detailed information on the instrument set see the guide by Max, and the instrument manuals which are downloadable from the Cambridge Instrument web site.

Glide Computer

The glide computer has many functions. Its main use is in calculating whether you have enough height to reach your next waypoint, and how fast you should be flying for the current conditions. To get to he main flying screen turn on the flight recorder using the volume knob (as you did in the first flight). Now press the go button followed by the right arrow.

The screen displayed should show two numbers on the top row. The first number is a calculated 30 second average of the vertical speed of the air, that is it will tell you whether you have been flying through air that is rising or sinking. The second number is a 30 second average of the rate of climb/sink of the glider. You should take of this number when you have been in lift for at least 30 seconds, it will tell you what your average rate of climb is. If you are climbing and this number starts to fall significantly it may may mean that the lift is getting weaker and it is time to move on (it may also mean you are getting tired and not flying as well).

At the bottom of the display is a number with MC before it. This is where you can enter the average rate of climb you expect to make at the next thermal. You can change this number using the up down arrow keys. While there are probably several different opinions as to what to set this to it should be set to a number less than the number you have been seeing on the average rate of climb number. I generally do not set this any higher than 5.0, you will need to experiment to see what works for you. Remember that the lift will weaken after lunchtime.

On the left of the display is a line with a triangle of horizontal lines either above of below it. This is telling you whether you need to fly faster or slower. If the horizontal lines are below the fixed line you need to put the nose down and fly faster, conversely if the horizontal lines are above the fixed line. The more lines the more you need to adjust your speed. When you are flying at the correct computed speed there will only be the fixed line.

On the right of the screen is another fixed line with a glider either above it or below it. This is an indication as to whether you have enough altitude to make the next waypoint, if the glider is above the line the computer thinks you have enough altitude, the further above the line the more spare altitude you have.

The two numbers in the centre of the screen are the distance to goal (top one) and difference between altitude required to reach next waypoint and current altitude, positive value means you have that much spare altitude. Beware, the instruments have no idea of the topography.

## Sending polars to the L-NAV

For the glide computer to be able to make its calculations it needs to know the polar of the aircraft you are flying. This is done using FSZWeverView. First you will need to get the polar. Kris Feldmann has measured a number of polars for the simulated aircraft, they are available at http://www.rotted.com/  While it is possible to enter the polar using FSZWeverView, I can never remember how to do it. I generally open the LNAV_polarlist.dat file in the fs9 root directory using notepad and add a line containing the new polar. I then open the polar page from the menu in FSZWeverView, use the bottom set of arrow keys to navigate to the aircraft I intend to fly and press the send button. You can check that this worked by looking at the LNAV_polar.dat file in the fs9 root directory to ensure it has the same numbers in it (it should be OK if you have FSZWeverView installed correctly.)

# Creating lift

I will give a brief overview on using Thermik here, again there is more detailed information available from Max’s site and in the SOAR library.

Thermik is used to create lift scenery files. The training package includes a few thermik project files to help get you started. The project file that was used to create the training flight is included, plus a couple of other project files in the surrounding area. I will use the project file included for the example flight here. Note the lift on the example file is not the same as the lift included in the training package BGL file. The example thermik project files are located in …\SOAR Scenery\Thermiek Projects directory.

First open the omarama.prj file in Thermiek, you should see a map with some red dots on it. The red dots are the thermals. To ensure that thermiek is working for you select Create->Create from the menu then press OK. You should now have a file “omarama_a.bgl” in the same directory. Replace the …\SOAR Scenery\Thermal BGLS\scenery\ omarama_a.bgl with the new file and start the training flight again, you should see fewer clouds in the sky, but there should still be some about with lift under them. Note you need to restart the simulator each time you change a scenery file.

Now go back to thermiek and and select Thermals->Clear all, you should now have a blank map. Now select Thermals->Manual. The window displayed has a few settings in it; Type 1 lift is wind dependent lift, its strength depends on the strength and direction of the wind. Type 2 and 3 lift are turbulence, they are similar to wind dependent lift except they are bumpy and not as strong. Type 4 lift is thermal lift, its strength depends on the time of the day, they are strongest around noon. The data section contains the bottom and top, which is the height above the ground for the top and bottom of the lift. The radius is half the width of the lift (the lift is actually square). The wind direction and factor are used for types 1-3 lift to set the wind direction that will cause the best lift and the strength of that lift respectively. The factor should be set to a value between 20000 and 30000 for reasonable lift. The cloud setting sets the parameters for a cloud above the thermal, if required. The ceiling is the height above ground of the cloud base, the radius is the size and the type is the bitmap that will be displayed.

Now setup the parameters for the type of lift you wish to create, press the set button and start clicking on the map. You should see thermal placed wherever you click. If you wish to zoom in / out you can use the number pad +/- keys. When you are happy you can save the project and create another bgl file.

The other project files included in the package are to the south west (milford) and north west (fox), allowing you to make a larger course to fly.

If you wish to make a project for another area you will need a map with grid lines on, preferably a mercator projection (straight grid lines). When you start the

# Creating Tasks

To create your own task you will first need to get a list of waypoints for the area you want to fly in. The simplest way to do this is to download some from the world turnpoint exchange at http://acro.harvard.edu/SSA/JL/TP/HomePage.html. You will need to get the .dat format for Cambridge Instruments GPS-NAV. The package includes a .dat file for the south island of New Zealand (where the training flight is based) which was downloaded from there.

To create a task first open the .dat file in FSZWeverView. Now select “Task->New” from the menu. You should now see all of the turnpoints and a blank task in the corner. If want to zoom in/out you can use the numeric keypad +/- keys. You should also have a list of waypoints down the right hand side, with the numbers and names. To create a task simply double click on the waypoints in the list on the right in the order you wish to fly them. The task should be drawn on the screen and the Task Selection in the top left corner will show the details. Once you have made the task you can send it to the GPS-NAV. If you wish to save the task file you will need to load it back from the GPS-NAV (“Load->From GPS-NAV”) and then save the flight data (first disk icon). This save both the flight data (.log file) and the task (.dat file). You can delete the .lof file if you do not need it.

# Problems with meshes

The accuracy and resolution of various meshes varies a lot. This can cause problems with wind dependent lift as the designer will generally place them on the ridge lines of the mesh that they are using, which may be in a different location to the one you were intending to use. If you are not find ridge lift where you think there should be try finding out what mesh the designer intended their scenery to be used with (after you make sure you have some wind set from the correct direction.)

# About the example flight

The example flight is set at Omarama, on the South Island of New Zealand. While it was designed using the default Microsoft mesh I generally use the mesh available at http://www.geographx.co.nz/Downloads.htm.

# What next

Once you have completed the training flights and Silver Certificate you should have enough skills to compete in any of the races that are currently happening, fly other certificate flights, e.g. the Gold certificate, or just enjoy some of the pre-made course that are available.

# Useful links

The SOAR site, http://www.virtualsoaring.org, has a lot of useful resources including;

The SOAR message board has lots of information and is a great place to ask any questions.

The SOAR Library which has a lot of useful information in it.

The Certificates and diploma pages which have some good course to fly

The Races pages which have many course that can be flown for fun.

The Scenery pages which have lift bgl’s for various parts of the world

Plus lots more…

Max’s page, http://www.fszwever.com,  which has aircraft, instruments and other usful utilities and information.

Wolfgains page, http://home.t-online.de/home/w.piper/homepage.htm, which has many aircraft available

Kris’s page, http://www.rotted.com, which has polars plus other information

The world turnpoint exchange, http://acro.harvard.edu/SSA/JL/TP/HomePage.html, wich has GPS files for all over the world